Stepper Motor Characterization and Missed-Step Detection System Using 8051 Microcontroller

by

RAKSHITH RAGAVAN S	24BMH1031

GOPIKRISHNA V	24BMH1123

A project report submitted to

Dr. M. JAGANNATH

SCHOOL OF ELECTRONICS ENGINEERING

in partial fulfilment of the requirements for the course of BMHA204L – Microprocessors and Microcontrollers in

B.Tech. Mechatronics and Automation

Vellore Institute of Technology (VIT) CHENNAI Vandalur – Kelambakkam Road

Chennai – 600127

## APRIL 2026

BONAFIDE CERTIFICATE

Certified that this project report entitled “Stepper Motor Characterization and Missed-Step Detection System Using 8051 Microcontroller” is a bonafide work of RAKSHITH RAGAVAN S (24BMH1031), GOPIKRISHNA V (24BMH1123) who carried out the Project work under my supervision and guidance.

### Dr. M. JAGANNATH

Professor

School of Electronics Engineering (SENSE), VIT University, Chennai

Chennai – 600 127.

## ABSTRACT

This project presents the design and implementation of a Stepper Motor Characterization and Missed-Step Detection System using an 8051 microcontroller. Stepper motors are widely used in precision control applications due to their ability to move in discrete steps without requiring continuous feedback. However, in practical systems, factors such as load variation, insufficient torque, and supply instability can cause missed steps, leading to inaccuracies in positioning. This project addresses that limitation by incorporating a feedback-based monitoring system.

The system consists of an 8051 microcontroller as the central control unit, a ULN2003 driver circuit to interface and drive the stepper motor, a rotary encoder for feedback, push buttons for speed control, and a 16×2 I2C LCD for real-time display. The stepper motor is powered independently using a 7.4V battery to ensure sufficient current supply, while the control circuitry including the microcontroller, encoder, and LCD is powered through a 5V USB source from a laptop. This separation of power sources enhances system stability and prevents noise interference.

The microcontroller generates stepping sequences to control the rotation of the motor using a half-step driving method, which improves smoothness and resolution. The speed of the motor is controlled using push buttons, which adjust predefined delay values stored in lookup tables. Corresponding RPM values are also mapped and displayed on the LCD.

To detect missed steps, the system compares the number of commanded steps generated by the microcontroller with the actual movement measured using a rotary encoder. The encoder is interfaced using an external interrupt pin, allowing accurate pulse counting in real time. The difference between commanded and actual steps is computed as missed steps, which provides a quantitative measure of system performance.

The LCD continuously displays key parameters such as motor speed (RPM), direction of rotation, and the number of missed steps. This enables real-time monitoring and analysis of the motor’s behavior under different operating conditions. Experimental observations show that as the speed increases, the

number of missed steps also increases due to reduced torque availability and dynamic limitations of the motor.

The system does not implement closed-loop correction but focuses on monitoring and analysis, making it suitable for educational and diagnostic purposes. It provides a simple, low-cost, and effective method for understanding stepper motor behavior and performance limitations.

In conclusion, this project successfully demonstrates the integration of control and feedback systems using an 8051 microcontroller to characterize stepper motor performance and detect missed steps, thereby improving reliability and providing valuable insights for real-world applications such as robotics, CNC machines, and automation systems.

## ACKNOWLEDGEMENT

We wish to express our sincere thanks and deep sense of gratitude to our project guide, Dr. M. JAGANNATH, Professor, School of Electronics Engineering, for his consistent encouragement and valuable guidance offered to us in a pleasant manner throughout the course of the project work.

We are extremely grateful to Dr. Karunamoorthy, Dean of the School of Mechanical Engineering, VIT Chennai, for extending the facilities of the School towards our project and for her unstinting support.

We express our thanks to our Head of the Department Dr. Saravankumar

for his support throughout the course of this project.

We also take this opportunity to thank all the faculty of the School for their support and their wisdom imparted to us throughout the course.

We thank our parents, family, and friends for bearing with us throughout the course of our project and for the opportunity they provided us in undergoing this course in such a prestigious institution.

RAKSHITH RAGAVAN                                                         GOPIKRISHNA

TABLE OF CONTENTS

| Serial No. | TOPICS | Pg no |
| --- | --- | --- |
| 1 | Abstract | 3 |
| 2 | Acknowledgment | 5 |
| 3 | Introduction | 7 |
| 4 | Block Diagram | 8 |
| 5 | Components Used | 9-10 |
| 6 | Working Principle | 10 |
| 7 | Software used | 10 |
| 8 | Code Explanation | 11 |
| 9 | Detailed Hardware connections | 11-12 |
| 10 | Mathematical Analysis | 12 |
| 11 | Flow of Execution | 12 |
| 12 | Experimental Results | 12-13 |
| 13 | Testing Procedure | 13 |
| 14 | Troubleshooting | 13 |
| 15 | Advantages | 13 |
| 16 | Applications | 13 |
| 17 | Limitations | 14 |
| 18 | Future Components | 14 |
| 19 | Conclusion | 14 |
| 20 | References | 14 |
| 21 | Appendix | 15-21 |

- INTRODUCTION

Stepper motors are widely used in applications that require precise position control such as CNC machines, robotics, and automation systems. Unlike conventional motors, stepper motors move in discrete steps, allowing accurate control without feedback in simple systems. However, in real-world conditions, missed steps can occur due to load variations, insufficient torque, or power limitations.

This project focuses on designing and implementing a system that not only controls a stepper motor using an 8051 microcontroller but also detects missed steps using an encoder feedback mechanism. The system also displays real-time information such as RPM and missed steps on an I2C LCD.

  - OBJECTIVES AND GOALS

The main objectives are:

- To control a stepper motor using an 8051 microcontroller

- To vary motor speed using push buttons

- To measure actual motor movement using a rotary encoder

- To detect missed steps by comparing commanded and actual steps

- To display RPM and missed steps on an LCD

## SYSTEM OVERVIEW

The system consists of three main sections:

- Control Section (8051 Microcontroller)

- Drive Section (ULN2003 + Stepper Motor)

- Feedback Section (Rotary Encoder)

Power is divided into two parts:

- 8051, LCD, and Encoder are powered using USB (5V)

- Stepper motor is powered using a 7.4V battery

This separation ensures stable operation and prevents voltage drops affecting the microcontroller.

- BLOCK DIAGRAM

The block diagram consists of the following components:

- Buttons provide user input for speed control

- 8051 processes inputs and generates control signals

- ULN2003 driver amplifies signals to drive the motor

- Stepper motor rotates based on signals

- Encoder provides feedback of actual motion

- LCD displays RPM and missed steps

- Components Used

  - 8051 Microcontroller

The 8051 acts as the brain of the system. It performs the following tasks:

- Reads button inputs

- Generates stepping sequence for motor

- Counts commanded steps

- Reads encoder pulses via interrupt

- Calculates missed steps

- Sends data to LCD

It is powered via USB from the laptop.

  - Stepper Motor (28BYJ-48)

This is a low-speed, high-precision motor. It operates in steps and requires a driver circuit. It is powered separately using a 7.4V battery to ensure sufficient current.

Working:

- Each input pulse rotates the motor by a fixed angle

- Half-step sequence is used for smoother motion

  - ULN2003 Motor Driver

ULN2003 is a Darlington transistor array used to drive the motor.

Functions:

- Amplifies low current signals from 8051

- Provides sufficient current to motor coils

- Protects microcontroller from back EMF

  - Rotary Encoder (KY-040)

The encoder provides feedback about actual rotation.

Working:

- Generates pulses when shaft rotates

- Each pulse corresponds to a movement

- Connected to interrupt pin of 8051

  - I2C LCD Display (16x2)

Used to display:

- RPM

- Missed steps

Advantages:

- Uses only 2 pins (SDA, SCL)

- Reduces wiring complexity

  - Push Buttons

Used for speed control:

- Increase speed

- Decrease speed

Pull-up resistors ensure stable input signals.

  - Power Supply

Two separate supplies are used:

- USB 5V for logic circuit

- 7.4V battery for motor

This prevents noise and voltage drops.

- Working Principle

- User sets speed using buttons

- 8051 selects delay value from table

- Stepper motor rotates using half-step sequence

- Each step increments commanded steps counter

- Encoder generates pulses for actual movement

- Interrupt updates encoder count

- Missed steps = commanded - actual

- LCD displays values periodically

- Software Used

- Keil uVision 5

Used for:

- Writing C code

- Compiling program

- Generating HEX file

- ProgISP

Used for:

- Uploading HEX file to microcontroller

- Code Explanation

Initialization

- IT1 = 1 → Edge triggered interrupt

- EX1 = 1 → Enable INT1

- EA = 1 → Global interrupt enable

Stepper Function

- Uses half-step sequence array

- Cycles through 8 states

Encoder ISR

- Triggered on every pulse

- Increments total_enc

Speed Control

- Buttons adjust speed_idx

- Lookup tables used for delay and RPM

LCD Update

- Converts numbers to string

- Displays RPM and missed steps

- Detailed Hardware Connections

- 8051 Pin Configuration

- P1.0 – IN1 (Stepper)

- P1.1 – IN2 (Stepper)

- P1.2 – IN3 (Stepper)

- P1.3 – IN4 (Stepper)

- P2.6 – SDA (I2C LCD)

- P2.7 – SCL (I2C LCD)

- P3.3 – Encoder CLK (INT1)

- P3.4 – Encoder DT

- P3.6 – Speed UP Button

- P3.7 – Speed DOWN Button

- Stepper + ULN2003

- IN1–IN4 from 8051 → ULN2003 inputs

- ULN2003 outputs → Stepper motor coils

- Motor powered by 7.4V battery

- Common GND with 8051

- LCD (I2C)

- VCC → 5V

- GND → GND

- SDA → P2.6

- SCL → P2.7

- Encoder

- VCC → 5V

- GND → GND

- CLK → P3.3

- DT → P3.4

- Mathematical Analysis

- Stepper Motor Resolution

For 28BYJ-48:

- Steps per revolution ≈ 2048 (half-step mode)

- RPM Calculation

RPM is approximated using lookup table:

RPM ≈ (steps per second × 60) / steps per revolution

- Missed Step Formula

missed = (commanded steps / 200) − encoder steps

Where:

- commanded steps = steps issued by controller

- encoder steps = actual movement detected

- Flow of Execution

- System initializes ports and LCD

- Interrupt enabled for encoder

- User sets speed using buttons

- Stepper rotates in loop

- Commanded steps counted

- Encoder ISR updates actual count

- Missed steps calculated

- LCD updated periodically

- Experimental Results

| Speed Index | RPM | Commanded Steps | Encoder Steps | Missed Steps |
| --- | --- | --- | --- | --- |
| 0 | 12 | 100 | 0 | 100 |
| 3 | 7 | 100 | 100 | 0 |
| 6 | 4 | 100 | 99 | 1 |
| 9 | 3 | 100 | 92 | 8 |

Observation:

- Higher speed → more missed steps

- Due to torque limitations

Sample LCD Outputs

Line 1: RPM:12 DIR:CW

Line 2: MISS:000002

- Testing Procedure

- Power the 8051 using USB

- Power motor using battery

- Upload code via ProgISP

- Observe LCD output

- Change speed using buttons

- Compare encoder vs commanded steps

- Troubleshooting

Problem: Motor not rotating

- Check ULN2003 connections

- Check battery voltage

Problem: LCD not displaying

- Check I2C address

- Check wiring

Problem: No encoder reading

- Check interrupt pin connection

- Advantages of the System

- Detects errors in real-time

- Improves reliability of stepper systems

- Low cost implementation

- Simple and effective design

- Applications

- CNC machines

- Robotics

- 3D printers

- Automated positioning systems

- Limitations

- Limited speed due to 8051

- Basic encoder accuracy

- No closed-loop correction

- Future Improvements

- Add PID control

- Use high-resolution encoder

- Implement wireless monitoring

- Conclusion

This project successfully demonstrates the control and performance analysis of a stepper motor using an 8051 microcontroller. The system is capable of generating accurate step sequences to drive the motor through the ULN2003 driver, while allowing user-controlled speed variation using push buttons.

A major outcome of the project is the ability to detect missed steps using a rotary encoder. By comparing the commanded steps from the microcontroller with the actual movement measured by the encoder, the system provides a clear indication of performance errors. This helps in understanding how factors such as speed and load affect motor accuracy.

The use of an I2C LCD enables real-time display of important parameters like RPM and missed steps, making the system easy to monitor and analyze. The separation of power supply for the motor and control circuit ensures stable operation and improves reliability.

Overall, the project provides a simple, low-cost, and effective method for studying stepper motor behavior and highlights the importance of feedback in motion control systems. It can be further improved by implementing closed-loop control techniques for automatic error correction.

- References

- 8051 Microcontroller Datasheet

- ULN2003 Datasheet

- Stepper Motor Documentation

- APPENDIX:

Software Implementation:

#include <reg52.h>

/* ---- Stepper coil pins ---- */

sbit IN1 = P1^0;

sbit IN2 = P1^1;

sbit IN3 = P1^2;

sbit IN4 = P1^3;

/* ---- I2C pins ---- */

sbit SDA = P2^6;

sbit SCL = P2^7;

/* ---- Encoder ---- */

sbit ENC_CLK = P3^3;

sbit ENC_DT  = P3^4;

/* ---- Buttons (active LOW, 100k pull-up to VCC) ---- */

sbit BTN_UP   = P3^6;

sbit BTN_DOWN = P3^7;

/* ==================== LCD CONFIG ==================== */

#define LCD_ADDR  0x27

#define LCD_BL    0x08

#define LCD_EN    0x04

#define LCD_RS    0x01

/* ==================== STEPPER CONFIG ==================== */

unsigned int  speed_table[10] = {1, 3, 5, 7, 9, 11, 13, 15, 17, 19};

unsigned char rpm_table[10]   = {12,10, 8, 7, 6,  5,  4,  4,  3,  3};

unsigned char speed_idx = 0;

unsigned char half_step[8] = {

0x09, 0x08, 0x0C, 0x04,

0x06, 0x02, 0x03, 0x01

};

unsigned char step_idx = 0;

/* ==================== COUNTERS ==================== */

unsigned long total_cmd = 0;

unsigned long total_enc = 0;

long          missed    = 0;

/* ==================== BUTTON STATE ==================== */

unsigned char up_state   = 1;

unsigned char down_state = 1;

unsigned char up_cnt     = 0;

unsigned char down_cnt   = 0;

/* ==================== DELAY ==================== */

void delay_1ms(void)

{

unsigned char j;

for (j = 0; j < 123; j++);

if (BTN_UP == 0) {

up_cnt++;

if (up_cnt >= 20 && up_state == 1) {

up_state = 0;

if (speed_idx > 0) speed_idx--;

}

} else {

up_cnt = 0;

up_state = 1;

}

if (BTN_DOWN == 0) {

down_cnt++;

if (down_cnt >= 20 && down_state == 1) {

down_state = 0;

if (speed_idx < 9) speed_idx++;

}

} else {

down_cnt = 0;

down_state = 1;

}

}

void delay_ms(unsigned int ms)

{

unsigned int i;

for (i = 0; i < ms; i++) delay_1ms();

}

void delay_us(unsigned char us)

{

unsigned char i;

for (i = 0; i < us; i++);

}

/* ==================== ENCODER ISR ==================== */

void encoder_isr(void) interrupt 2

{

total_enc++;

}

/* ==================== I2C ==================== */

void i2c_start(void)

{

SDA = 1; SCL = 1; delay_us(5);

SDA = 0; delay_us(5);

SCL = 0; delay_us(5);

}

void i2c_stop(void)

{

SDA = 0; SCL = 1; delay_us(5);

SDA = 1; delay_us(5);

}

void i2c_write_byte(unsigned char val)

{

unsigned char i;

for (i = 0; i < 8; i++) {

SDA = (val & 0x80) ? 1 : 0;

val <<= 1;

SCL = 1; delay_us(5);

SCL = 0; delay_us(5);

}

SDA = 1;

SCL = 1; delay_us(5);

SCL = 0; delay_us(5);

}

/* ==================== LCD ==================== */

void lcd_send_nibble(unsigned char nibble, unsigned char rs)

{

unsigned char dat_byte;

dat_byte = (nibble << 4) | LCD_BL | (rs ? LCD_RS : 0);

i2c_start();

i2c_write_byte(LCD_ADDR << 1);

i2c_write_byte(dat_byte | LCD_EN);

delay_us(1);

i2c_write_byte(dat_byte & ~LCD_EN);

delay_us(50);

i2c_stop();

}

void lcd_send_byte(unsigned char byte_val, unsigned char rs)

{

lcd_send_nibble(byte_val >> 4, rs);

lcd_send_nibble(byte_val & 0x0F, rs);

}

void lcd_cmd(unsigned char cmd)

{

lcd_send_byte(cmd, 0);

delay_ms(2);

}

void lcd_char(unsigned char ch)

{

lcd_send_byte(ch, 1);

}

void lcd_init(void)

{

delay_ms(50);

lcd_send_nibble(0x03, 0); delay_ms(5);

lcd_send_nibble(0x03, 0); delay_ms(1);

lcd_send_nibble(0x03, 0); delay_ms(1);

lcd_send_nibble(0x02, 0);

lcd_cmd(0x28);

lcd_cmd(0x0C);

lcd_cmd(0x06);

lcd_cmd(0x01);

delay_ms(2);

}

void lcd_set_cursor(unsigned char row, unsigned char col)

{

lcd_cmd(0x80 | (row ? 0x40 : 0x00) | col);

}

void lcd_print(char *str)

{

while (*str) lcd_char(*str++);

}

void int_to_str(long val, char *buf, unsigned char width)

{

char tmp[12];

unsigned char i=0, j=0, neg=0;

if (val < 0) { neg = 1; val = -val; }

if (val == 0) tmp[i++] = '0';

while (val > 0) { tmp[i++] = '0' + (char)(val % 10); val /= 10; }

if (neg) tmp[i++] = '-';

while (j < (unsigned char)(width - i)) buf[j++] = ' ';

while (i > 0) buf[j++] = tmp[--i];

buf[j] = '\0';

}

/* ==================== LCD UPDATE ==================== */

void lcd_update(void)

{

char buf[8];

lcd_set_cursor(0, 0);

lcd_print("RPM:");

int_to_str((long)rpm_table[speed_idx], buf, 2);

lcd_print(buf);

lcd_print("  DIR:CW");

// ? FIXED CALCULATION ONLY

missed = (long)(total_cmd / 200) - (long)total_enc;

if (missed < 0) missed = 0;

lcd_set_cursor(1, 0);

lcd_print("MISS:");

int_to_str(missed, buf, 6);

lcd_print(buf);

lcd_print("  ");

}

/* ==================== STEPPER ==================== */

void stepper_step(void)

{

step_idx = (step_idx + 1) & 0x07;

P1 = (P1 & 0xF0) | half_step[step_idx];

total_cmd++;

}

void stepper_off(void)

{

P1 &= 0xF0;

}

/* ==================== MAIN ==================== */

void main(void)

{

unsigned int i;

unsigned int steps_per_batch = 512;

unsigned char prev_speed = 0xFF;

IT1 = 1;

EX1 = 1;

EA  = 1;

BTN_UP = 1;

BTN_DOWN = 1;

ENC_CLK = 1;

ENC_DT  = 1;

SCL = 1;

SDA = 1;

total_cmd = 0;

total_enc = 0;

missed = 0;

lcd_init();

lcd_update();

delay_ms(1000);

while (1)

{

for (i = 0; i < steps_per_batch; i++)

{

stepper_step();

delay_ms(speed_table[speed_idx]);

if (speed_idx != prev_speed) {

prev_speed = speed_idx;

lcd_update();

}

}

stepper_off();

delay_ms(50);

lcd_update();

delay_ms(1500);

}

}